Autonomous Mobile Robot with LiDAR Mapping

Emir Orozbekov

Hof University of Applied Sciences Hof, Germany

[emir.orozbekov@hof-university.de]

Altynai Munduzbaeva

Hof University of Applied Sciences Hof, Germany

[altynai.munduzbaeva@hof university.de]

Kyial Asykpaeva

Hof University of Applied Sciences Hof, Germany

[kyial.asykpaeva@hof-university.de]

Zhanetta Tynymbekova Hof University of Applied Sciences Hof, Germany

[zhanetta.tynymbekova@hof university.de]

Hanna Hryshyna

Hof University of Applied Sciences Hof, Germany

[hanna.hryshyna@hof-university.de]

Nargiza Alieva

Hof University of Applied Sciences Hof, Germany

[nargiza.alieva@hof-university.de]

Abstract

Autonomous mobile robots are increasingly used in indoor environ ments such as research laboratories, service robotics, and logistics systems. A key challenge for such robots is the ability to perceive unknown environments, detect obstacles, and navigate safely with out human intervention.

This paper presents the design and implementation of an au tonomous mobile robot based on classical robotics approaches using LiDAR-based perception and the Robot Operating System (ROS). The system is built on a UGV02 tracked mobile platform and uses an RPLidar A1 sensor for real-time environment scanning and map ping. Simultaneous Localization and Mapping (SLAM) is employed to construct an occupancy grid map of an unknown indoor envi ronment while estimating the robot’s position.

The proposed system demonstrates reliable obstacle detection, real-time map generation, and autonomous navigation. Experimen tal results show that the robot is able to distinguish free space, obstacles, and unknown areas with sufficient accuracy for indoor navigation tasks. The project emphasizes system integration, mod ular software design, and robustness rather than machine learning based approaches.

Keywords

Autonomous Mobile Robot, LiDAR, ROS, SLAM, Obstacle Avoid ance, Indoor Mapping

ACM Reference Format:

Emir Orozbekov, Kyial Asykpaeva, Hanna Hryshyna, Altynai Munduzbaeva, Zhanetta Tynymbekova, and Nargiza Alieva. 2026. Autonomous Mobile Robot with LiDAR Mapping. In University Robotics Project, Hof University of Applied Sciences. ACM, New York, NY, USA, 2 pages. https://doi.org/ XXXXXXX.XXXXXXX

Permission to make digital or hard copies of all or part of this work for personal or classroom use is granted without fee provided that copies are not made or distributed for profit or commercial advantage and that copies bear this notice and the full citation on the first page. Copyrights for components of this work owned by others than the author(s) must be honored. Abstracting with credit is permitted. To copy otherwise, or republish, to post on servers or to redistribute to lists, requires prior specific permission and/or a fee. Request permissions from permissions@acm.org.

Robotics Project, Hof, Germany

© 2026 Copyright held by the owner/author(s). Publication rights licensed to ACM. https://doi.org/XXXXXXX.XXXXXXX

1 Introduction

Autonomous mobile robots play an increasingly important role in modern applications such as indoor logistics, service robotics, and research environments. A key requirement for such systems is the ability to perceive the environment, detect obstacles, and navigate safely without human intervention.

The goal of this project was to design and implement an au tonomous mobile robot capable of building a map of an unknown indoor environment using LiDAR-based perception and ROS. In stead of focusing on machine learning approaches, the project em phasizes classical robotics methods, system integration, and reliable real-time operation.

The robot is based on the UGV02 tracked mobile platform and uses a Jetson Xavier Nano Developer Kit as its onboard computer. Environment perception is achieved using an RPLidar A1 sensor, enabling real-time mapping and obstacle detection.

2 Background and Theory

2.1 Robot Operating System (ROS)

The Robot Operating System (ROS) is a widely used framework in robotics that provides tools and libraries for building complex robot software systems. It is based on a modular architecture where individual components, called nodes, communicate with each other using a publish–subscribe mechanism.

This communication model allows sensor processing, mapping, and control logic to be developed independently while still working together as a single system.

2.2 LiDAR-Based Perception

LiDAR (Light Detection and Ranging) sensors measure distances by emitting laser beams and calculating the time it takes for the reflected light to return. A 2D LiDAR sensor provides a planar scan of the environment, which is especially suitable for indoor navigation.

Compared to camera-based perception, LiDAR is independent of lighting conditions and provides accurate distance measurements, making it reliable for obstacle detection and mapping tasks.

Robotics Project, 2026, Hof, Germany Emir Orozbekov, Kyial Asykpaeva, Hanna Hryshyna, Altynai Munduzbaeva, Zhanetta Tynymbekova, and Nargiza Alieva

2.3 SLAM and Occupancy Grid Mapping Simultaneous Localization and Mapping (SLAM) refers to the prob lem of building a map of an unknown environment while simulta neously estimating the robot’s position within that map.

In this project, an occupancy grid representation is used, where the environment is divided into discrete cells representing free space, obstacles, or unknown areas. As the robot moves and collects LiDAR data, the map is continuously updated in real time.

3 System Architecture

3.1 Hardware Architecture

The hardware setup of the robot consists of the following main components:

• UGV02 tracked robot platform providing stable indoor move ment

• Jetson Xavier Nano Developer Kit serving as the onboard computer

• RPLidar A1 for distance measurement and environment scan ning

• Motor controller and motors responsible for robot movement • Onboard battery system supplying power to all components

All components are mounted on the robot platform and con nected via wired and wireless interfaces.

3.2 Software Architecture

The software architecture is built entirely on ROS and follows a modular design. The main components include a LiDAR node pub lishing laser scan data, a mapping node generating an occupancy grid map, a control and navigation node performing obstacle detec tion and decision-making, and visualization tools using RViz.

This architecture allows a clear separation of sensing, mapping, and control, which simplifies debugging and future system exten sions.

4 Implementation

4.1 Sensor Data Flow

The LiDAR sensor continuously publishes laser scan data to the ROS topic /scan. This data stream is consumed by both the mapping component and the obstacle detection logic.

4.2 Mapping Process

A SLAM-based approach is used to build the environment map while the robot moves. As new LiDAR scans arrive, they are inte grated into the map, gradually forming a consistent representation of the indoor environment.

4.3 Obstacle Detection and Control Logic Obstacle detection is performed directly using LiDAR scan data. Based on predefined safety thresholds, the robot decides whether to move forward, turn to avoid an obstacle, or stop in emergency situations.

4.4 Operating Modes

The robot supports three operating modes: STOP, MAP, and AUTO. This separation proved useful during development and testing.

5 Results

The implemented system demonstrates stable LiDAR data acqui sition, real-time SLAM-based map generation, reliable obstacle detection, and autonomous navigation in indoor environments.

6 Challenges and Limitations

Several challenges were encountered during the project, including limited battery life, a constrained indoor test environment, and the complexity of ROS configuration and hardware integration.

7 Conclusion and Future Work

This project demonstrates that a reliable autonomous mobile robot can be built using classical robotics techniques and careful system integration. Future work includes improved power management, additional sensors, and more advanced navigation strategies.

References

[1] M. Quigley et al., ‘ROS: an open-source Robot Operating System,” ICRA Workshop on Open Source Software, 2009.

[2] S. Thrun, W. Burgard, D. Fox, Probabilistic Robotics, MIT Press, 2005. [3] Slamtec, ‘RPLIDAR A1 Datasheet,” 2023.

[4] H. Durrant-Whyte and T. Bailey, “Simultaneous localization and mapping,” IEEE Robotics & Automation Magazine, 2006.